Brighton Main Line Route

Utilisation Strategy

February 2006

Brighton Main Line Route

Utilisation Strategy

Department for Transport

Great Minster House

76 Marsham Street

London SW1P 4DR

Telephone 020 7944 8300

Internet service: www.dft.gov.uk

© Crown Copyright 2006.

Copyright in the typographical arrangement and design vests in the Crown.

This publication (excluding the Royal Arms and logos) may be reproduced free of charge in any format

or medium provided that it is reproduced accurately and not used in a misleading context. The material

must be acknowledged as Crown copyright with the title and source of the publication specified.

This document is available on the DfT website: www.dft.gov.uk

Published by the Department for Transport.

Printed in the UK February 2006 on paper containing 80 per cent post consumer waste and

20 per cent TCF pulp.

Product code 56RAIL02519

CONTENTS

Page

Foreword 5

Executive summary 6

1. The Context of the Brighton Main Line Route Utilisation Strategy 10

2. Consultation Period 13

3. Baseline information 17

4. Drivers of change – Stakeholder issues 29

5. Drivers of change – Growth 39

6. Drivers of change – Commitments and affordability 45

7. Strategy 47

Appendix A: Appraisal methodology 55

Appendix B: List of formal consultees 56

Glossary 57

Foreword

The Brighton Main Line Route Utilisation Strategy (RUS) sets out how forecast passenger

and freight demand on this important route can be accommodated in the years before the

implementation of the Thameslink Programme.

It identifies the most cost-effective way to build further on the growth in use of rail from

London to, and within, Surrey and Sussex, including Gatwick Airport.

The consultation draft of this strategy was published by the Strategic Rail Authority (SRA)

in September 2004. This final document is published by the Department for Transport,

although it draws heavily on the work undertaken by SRA before it handed over its

strategic planning duties to DfT in June 2005. Ministers and DfT will accord it the same

status as the earlier Route Utilisation Strategies that were published by SRA. Future RUSs

will be taken forward by Network Rail, with the objective of creating a coherent set of

appraisals of all those parts of the network where questions of utilisation arise. Matters

that are of strategic importance, or relate to rail passenger franchising, will be addressed

by DfT Rail.

Thanks are due to those many organisations and individuals who took the time, and made

the effort, to make comments on the draft of this strategy, attended meetings, and put

forward their views. These views have been given serious reflection and a number of

changes have been made as a result.

A number of organisations have been involved in the RUS process from the outset,

helping guide the issues that form the basis of the RUS. This has been a very

constructive, inclusive, process and reflects the keen interest of many within Sussex and

Surrey to get the best for their rail services.

Department for Transport

February 2006

5

Executive summary

In September 2004, the SRA published, for consultation, its draft Route Utilisation

Strategy for the Brighton Main Line. This new document, published by the Department for

Transport, presents the final Strategy for the use of train and network capacity on this

important route from London (Victoria and London Bridge) via Croydon and Gatwick to the

Sussex Coast, concentrating on addressing the crowding and reliability issues on the

long-distance commuter services, whilst retaining features designed to meet the needs of

air travellers.

Prioritising main line changes

This final strategy focuses on changes to main line services. It is on the main line that the

SRA’s analysis has shown that the greatest benefits, in terms of financial betterment,

performance, and overcrowding, can be achieved. Early implementation of these features

of the strategy is therefore recommended.

On the other hand, early implementation of the draft strategy proposals for the suburban

area is not recommended. This is partly to allow Transport for London (TfL) and the Train

Operator further time to consider options, and partly because the draft recommendations

did not deliver the same level of financial benefit or overcrowding relief for the suburban

routes as for the main line routes. TfL now plays an increased role in service specification

in London, and the conclusions of this strategy are expected to inform TfL’s Route

Corridor Plans, which, in turn, will contribute to future franchise specifications.

The benefits

The Strategy will deliver the following benefits:

• A significant improvement in performance (punctuality) for all passengers

• An increase in peak-time train capacity for commuters

• Faster journey times between key Sussex towns and London

• A fast, frequent, high-quality service between Gatwick Airport and London

• Protection of the capacity required by freight operators

These significant benefits can be achieved within the constraints of the existing

infrastructure and the size of the current rolling stock fleets. They are designed to deliver

an affordable match of capacity to demand and to improve the performance of the railway.

Why this strategy is needed

The Route Utilisation Strategy for the Brighton Main Line addresses three primary

concerns:

Brighton Main Line Route Utilisation Strategy

6

• Punctuality. Although there have been some recent signs of improvement,

performance remains bedevilled by reactionary delays. The complex structure of

the timetable is a significant contributor to these delays.

• Capacity and overcrowding. There are very large commuter flows, dominated by

the London market, but the passenger loads are unevenly spread across the peak

train service, particularly to and from Victoria.

• Value for Money. The inefficient use of trains and network means that the cost of

supporting the franchised services is unnecessarily high.

The Strategy sets out the conclusions that have been reached in the quest to balance the

requirement to improve performance with the need to match service patterns better to

passenger demand. In proposing changes, it seeks solutions that are affordable, and

which give best value to both taxpayers and railway users. The Strategy is focused on the

medium term, defined here as 2006 to 2012, and advocates solutions which make best

use of the existing railway infrastructure.

Commitments already delivered

Certain schemes have been treated in the RUS as ‘baseline’, because work is already

complete, including:

• Power supply upgrade to allow the introduction of a new fleet of trains.

• Replacement of the electric slam-door Mark 1 rolling stock in its entirety with

higher-capacity class 377 Electrostars.

• The construction of the new and upgraded rolling stock maintenance facilities at

Brighton, Littlehampton, Eastbourne, Bognor Regis, Selhurst and Streatham Hill.

• The introduction of an hourly direct service from Uckfield to London Bridge,

operated by the new class 171 diesel units that have been procured to replace

the aged slam-door diesel units.

• The introduction of an hourly direct service from Brighton to Ashford, also

operated by the new class 171 units.

Changes to the strategy following consultation

During the consultation period, a large number of responses were received which,

depending on their origin, represented specific concerns, or provided broader comments

on the context and thrust of the strategy. The level and detail of the responses received

was pleasing, and thanks are due to all those individuals and organisations which

participated.

Many months have been spent assimilating the consultation responses, and making

changes to the specification as appropriate. We have also spent further time with Network

Executive summary

7

Rail, SEERA, local authorities, TfL, the Rail Passengers’ Council, LTUC (now London

TravelWatch), the Train Operators, Airlines and BAA, working through the detail of our

plans. Particular emphasis has been placed on the evaluation of the many suggestions

and concerns relating to proposals for the Gatwick Express, especially in respect of the

handling of morning peak loads.

The Consultation chapter of this document provides detail on the content and concerns

expressed during the public consultation. Where specific errors or omissions that are

material to the strategy have been brought to our attention, corrections have been

recorded in the Consultation chapter.

The majority of responses supported the recommendations made in the consultation draft,

but in recognition of the concerns particularly of BAA and the airlines, we have adopted, in

this finalised strategy, a more flexible approach to the service between London, Gatwick

and the Sussex Coast.

In particular, the train service on this route has been assessed in three segments:

• Sussex Coast to Victoria

• Sussex Coast to London Bridge and Thameslink

• Wider Gatwick area (Crawley/Gatwick/Redhill) to Victoria

Analysis of the timetable options and performance data confirms that the maximum

number of main line train slots consistent with a robust timetable is 18 per hour – provided

that slow/fast line crossing movements are minimised on the 4-track sections of the route.

However, the level of track capacity consumed by 18 trains per hour (tph) leaves very little

margin for recovery, and can only be sustained for the short periods necessary to support

the high level of passenger demand experienced in the commuting peaks.

This Strategy therefore recommends that the operation should be restricted to a lower

level through most of the day – ideally no more than 14tph, rising to 18tph only in the peaks.

This level of service, however, precludes the operation of some consultees’ preferred

calling patterns – trains must make more intermediate calls than some consider ideal.

The choice between the 14tph and 18tph solutions (and the possibilities in between), and

the impact on calling patterns, provides a classic example of the trade-off between route

capacity and timetable resilience – one of the main reasons for the instigation of RUS

studies. Network Rail will carry out detailed performance modelling of these options

before the timetable work is completed.

Whatever the outcome of this modelling might recommend, four train paths in each offpeak

hour will be allocated to Thameslink services, with the remainder shared between

services to/from the Sussex Coast, and trains that start/terminate in the Gatwick area.

The concept of a fast, frequent service between Gatwick and Victoria will remain, and,

although it will be necessary for some of these trains to serve the wider Gatwick area, and

Brighton Main Line Route Utilisation Strategy

8

some intermediate stations between Gatwick and London, they will retain a high-quality

customer service specification designed to meet the specific airline market need.

We are confident that the revised Strategy will ensure that Gatwick Airport will get an

improved train service (because most, if not all, of the Sussex Coast trains will also serve

Gatwick) while securing the wide-ranging performance and capacity benefits for other

users along the route. This is a policy with much greater flexibility than the existing

arrangement, or that proposed in the RUS draft.

It has not been possible to meet all the aspirations of stakeholders, as in some cases they

conflict – either with each other, or with the objectives for Capacity Utilisation work

regarding value for money and affordability. To achieve the best possible service for the

majority of passengers, there are a number of changes that need to be made:

• Because of the enhancements proposed for the service from coastal towns to

Gatwick, and to ensure better performance for all trains in the Gatwick area, it will

not be possible for there always to be a Victoria-bound train waiting at the

platform at Gatwick Airport.

• In accordance with the path allocation priorities listed above, and the requirement

to minimise the fast/slow line crossing manoeuvres, this Strategy proposes that

the existing Watford Junction to Brighton trains should not run south of Clapham

Junction on the fast lines. This service alteration both avoids the conflicting train

movements between slow and fast lines in the Balham area, and also reduces

main line route congestion. It is acknowledged that there may be alternative

useful destinations for the Watford trains, accessible via the slow lines from

Clapham Junction, and that it is particularly desirable to identify a solution that

maintains the link with East Croydon.

• In order to deliver better performance along Coastway East, and to make more

cost-effective use of the resources available to the railway industry, the all-day

service from Hastings and Bexhill to London (via Gatwick) is now proposed to

start and terminate at Eastbourne. However, in response to comments received

during consultation, a number of through morning and evening peak services

between Hastings/Bexhill and Gatwick/Croydon/London Victoria will be retained.

Delivery of the strategy

The timing and delivery of this Strategy will be dependent on negotiations with industry

partners. DfT are working with Network Rail, BAA, and the incumbent operators,

particularly Southern, Gatwick Express and Thameslink, to bring about the improvements

described in this Strategy. This may entail some alterations to existing agreements,

including Franchise Agreements.

The detailed timetable work, as on the remainder of the network, will be undertaken by

Network Rail, taking cognisance of proposals made by the Train Operators, and subject to

the strategic overview taken by the Department for Transport.

Executive summary

9